TRAFFIC INTERSECTION DISPLAY PANEL

## PART # TDC-106 DOGHOUSE ### CHASSIS 30” X 30” X 2 BLACK ANODIZED ALUMINUM TRAFFIC PATTERN SILKSCREEN 28” X 28” DISPLAYED OUT PUTS 8 PHASES (RED-YEL-GRN) 6 OVERLAPS (RED-YEL-GRN) 4 PEDESTRIAN (ORG-BLU-WHT) 2 ADVANCED WARNING (ORG) INCANDESANT DISPLAY LAMPS 4 WATTS 130VAC ### CABLE #### (ORDER SEPARATELY) PART # TDC-260 8FT. TERMINATED IN SPADE LUGS (FOR OUTPUT FILE TERMINALS) (PHOENIX BLOCK COMPATABLE) The TDC-106 (Doghouse) Display Panel with cable TDC-260 (8 ft) allows the displaying of the outputs from a Traffic Cabinet. Red, Yellow, and Green lamps are used to display the 8 Traffic Phases as well as the 6 overlaps. The active pedestrian lamps are White (Walk) and Amber (Don’t Walk). The non-active outputs (Pedestrian yellow and Doghouse red) are displayed at the top of the panel in blue. The panel is equipped with 2-keyhole type mounting slots located on the back near the top. These slots are on 16-inch centers. The TDC-106 (Doghouse) Display can be driven directly from a Traffic Control (170 Type) by utilizing TDC-109 interface. The TDC-106 will display the watchdog and the flash outputs of the controller.